CPE301 – SPRING 2019

# Design Assignment DA4a

Student Name: Daniel Senda

Student #: 5002362633

Student Email: sendad1@unlv.nevada.edu

Primary Github address: https://github.com/dsenda/Smiles

Directory: DA4a

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used:

Atmel Studio 7

ATmega328P Xplained Mini

DC motor driver module (TB6612FNG)

DC motor

Multi-function shield

Potentiometer (Multi-function shield)

Breadboard

Power Supply

Block diagram about the transmission connections:

ATmega328

On/Off Switch

Potentiometer

DC Motor

- INITIAL/MODIFIED CODE OF TASK DA4a

Not applicable, there is no modified code.

- DEVELOPED MODIFIED CODE OF TASK DA4a

The following is the code that accomplishes the desired tasks that are required.

// c_code_dc_motor_on_off_duty_cycle.c

// Daniel Senda

// Turn DC motor on and off with pushbutton and

// also changes speed with change of potentiometer.

#define	F_CPU 16000000UL

#include	<avr/io.h>

#include	<util/delay.h>

#include	<avr/interrupt.h>

// Declares global variables.

int motor = 0;	// On_Off variable for motor.

int poten = 0;	// Potentiometer variable.

// Declares function.

void adc_init(void);

int main(void)

{

DDRD = 0b00100000;	// Sets PD5 as an output.

DDRC = 0b00000000;	// Sets all pins in PORTC as inputs.

TCCR2A = 0x00;	// Sets normal mode on timer0.

TCCR2B = 0x01;	// Sets no pre-scaling.

TIMSK2 = 0x01;	// Enables timer0 overflow interrupt.

sei ();		// Enables global interrupts.

TCCR0A = 0b10100011;//Sets Fast PWM.

TCCR0B = 0x01;	// Sets no pre-scaling.

OCR0B = 0xCC;		// Sets 80% or 20% duty cycle.

adc_init();		// Initializes ADC.

while (1)

{

ADCSRA|=(1<<ADSC);			// Starts conversion.

while((ADCSRA&(1<<ADIF))==0);	// Waits for conversion to finish.

ADCSRA |= (1<<ADIF);			// Resets conversion finished flag.

poten = ADCL;				// Records potentiometer data.

poten = poten | (ADCH<<8);		// Data calculations.

poten = (poten/1024.0) * 5000/10;

OCR0B = poten;			// Sets OCR0B to poten value.

}

}

ISR (TIMER2_OVF_vect)		// timer0 overflow interrupt vector

{

if ((PINC & 0b10) == 0)	// If pushbutton is pressed.

{

motor ^= 1;			// Toggles motor variable.

if (motor == 1)		// Turns motor on.

{

TCCR0A = 0b00100011;

}

else				// Turns motor off.

{

TCCR0A = 0b00000011;

}

_delay_ms(500);		// Delay for de-bouncing.

}

}

void adc_init (void)	// Sets up and enables ADC.

{

ADMUX = (0<<REFS1)|	// Reference Selection Bits.

(1<<REFS0)|		// AVcc - external cap at AREF.

(0<<ADLAR)|		// ADC Left Adjust Result.

(0<<MUX2)|		// Analog Channel Selection Bits.

(0<<MUX1)|		// ADC0 (PC0).

(0<<MUX0);

ADCSRA = (1<<ADEN)|	// ADC Enable.

(0<<ADSC)|		// ADC Start Conversion.

(0<<ADATE)|		// ADC Auto Trigger Enable.

(0<<ADIF)|		// ADC Interrupt Flag.

(0<<ADIE)|		// ADC Interrupt Enable.

(1<<ADPS2)|		// ADC Pre-scaler Select Bits.

(0<<ADPS1)|

(1<<ADPS0);

}

- SCHEMATICS

The following schematics from the slides describe the connections that are made relatively well.

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

There are no required Atmel studio outputs, but below is an included image of Atmel.

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

The following is a picture of the board setup:

- VIDEO LINKS OF EACH DEMO

https://youtu.be/II7SXncMUsE

- GITHUB LINK OF THIS DA

https://github.com/dsenda/Smiles/tree/master/DA4a

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Daniel Senda