CPE301 – SPRING 2019

# Design Assignment DA5

Student Name: Daniel Senda

Student #: 5002362633

Student Email: sendad1@unlv.nevada.edu

Primary Github address: https://github.com/dsenda/Smiles

Directory: DA5

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used:

Atmel Studio 7

ATmega328P Xplained Mini

LM34 Temperature Sensor

FTDI Basic chip

NRF24L01+ RF Module

Breadboard

Jumper wires

Classmates set-up in same channel

Block diagram about the transmission connections:

LM34 Temperature

Sensor #1

ATmega328p

#1

RF Module

#1

RF Module

#2

ATmega328p

#2

LM34 Temperature

Sensor #2

- INITIAL/MODIFIED CODE FROM TASK DA3b

// c_code_temperature_on_serial2usb.c

// Daniel Senda

// Monitors LM34 to display temperature on serial terminal every 1s.

// Uses a timer with interrupt for the 1s delay.

#define	F_CPU 16000000UL	// Needed Definitions.

#define	BAUD_RATE 9600

#include	<avr/io.h>		// Needed Libraries.

#include	<util/delay.h>

#include	<avr/interrupt.h>

volatile int O_F = 0;		// Used for overflow count.

int num = 0;				// Used for while loops.

void usart_init ();

void usart_send (unsigned char ch);

void initiate_timer()

{

TCCR0A = 0b10;		// Sets CTC mode.

TCCR0B = 0b00000101;		// Sets 1024 pre-scaler.

OCR0A = 255;			// Sets output compare register A to 255.

TIMSK0 |= (1 << OCIE0A);	// Enables output compare match interrupt.

sei();				// Enables global interrupts.

TCNT0 = 0;			// Resets timer.

}

ISR(TIMER0_COMPA_vect)		// Keeps count of overflows.

{

O_F++;				// Increments overflow count.

}

int main (void)

{

usart_init ();

// Setup and enable ADC

ADMUX = (0<<REFS1)|		// Reference Selection Bits.

(1<<REFS0)|			// AVcc - external cap at AREF.

(0<<ADLAR)|			// ADC Left Adjust Result.

(1<<MUX2)|			// Analog Channel Selection Bits.

(0<<MUX1)|			// ADC4 (PC5).

(1<<MUX0);

ADCSRA = (1<<ADEN)|		// ADC ENable.

(0<<ADSC)|			// ADC Start Conversion.

(0<<ADATE)|			// ADC Auto Trigger Enable.

(0<<ADIF)|			// ADC Interrupt Flag.

(0<<ADIE)|			// ADC Interrupt Enable.

(1<<ADPS2)|			// ADC Pre-scaler Select Bits.

(0<<ADPS1)|

(1<<ADPS0);

initiate_timer();		// Function that initiates timer.

while (1)

{

ADCSRA|=(1<<ADSC);			// Starts conversion.

while((ADCSRA&(1<<ADIF))==0);	// Waits for conversion to finish.

ADCSRA |= (1<<ADIF);

num = 0;				// Resets num count.

TCNT0 = 0;				// Resets timer.

while (num <= 0)

{

if (O_F >= 61)

{

int a = ADCL;

a = a | (ADCH<<8);

a = (a/1024.0) * 5000/10;

usart_send((a/100)+'0');

a = a % 100;

usart_send((a/10)+'0');

a = a % 10;

usart_send((a)+'0');

usart_send('\r');

num++;			// Increments num to exit while loop.

O_F = 0;		// Resets overflow count.

}

}

}

return 0;

}

void usart_init (void)			// Initializes USART.

{

UCSR0B = (1<<TXEN0);

UCSR0C = (1<< UCSZ01)|(1<<UCSZ00);

UBRR0L = F_CPU/16/BAUD_RATE-1;

}

void usart_send (unsigned char ch)		// Transmits characters to computer.

{

while (! (UCSR0A & (1<<UDRE0)));	// Waits until UDR0 is empty.

UDR0 = ch;				// Transmit ch.

}

void usart_print(char* str)			// Prints characters on computer.

{

int i = 0;

while(str[i] != 0)

{

usart_send(str[i]);

}

}

- DEVELOPED MODIFIED CODE OF TASK DA5

// c_code_RF_temperature_serial2usb.c

// Daniel Senda

// Interface the provided NRF24L01+ RF Module to the ATmega328p using the SPI interface.

// Using the earlier developed code for ADC, transmit the ADC value of the internal

// temp sensor, or LM35 sensor between two RF Modules. The ATmega328p interfacing the

// RF Modules should alternate between TX and RX modes every 0.5 secs (hopefully they are

// not both at TX and RX modes in the same interval). The temp of both ATmega328p’s

// should be displayed on both ATmega328p’s.

#ifndef F_CPU					// Sets clock frequency.

#define F_CPU 16000000UL

#endif

#include <avr/io.h>				// Includes needed libraries.

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdbool.h>

#include <stdio.h>

#include <string.h>

#include "inc\nrf24l01.c"			// Include nRF24L01+ library.

#include "inc\nrf24l01-mnemonics.h"

#include "inc\spi.c"

#ifndef BAUD					// Sets up UART for printf();

#define BAUD 9600

#endif

#include "inc\STDIO_UART.c"

void print_config(void);			// Functions

void adc_init(void);

volatile bool message_received = false;	// Used in IRQ ISR.

volatile bool status = false;		// Used in IRQ ISR.

int tempf = 0;				// Used for temperature data.

int main(void)

{

adc_init();				// Initializes the ADC.

char tx_message[32];			// Defines string array.

strcpy(tx_message,"Hello World!");	// Copies string into array.

uart_init();			// Initializes UART.

nrf24_init();			// Initializes nRF24L01+ and print configuration info.

print_config();		// Configures prints.

nrf24_start_listening();			// Start listening to incoming messages.

nrf24_send_message(tx_message);		// Sends message.

while (1)

{

ADCSRA|=(1<<ADSC);			// Starts conversion.

while((ADCSRA&(1<<ADIF))==0);	// Waits for conversion to finish.

ADCSRA |= (1<<ADIF);			// Resets flag for conversion.

tempf = ADCL;				// Records temp sensor data.

tempf = tempf | (ADCH<<8);

tempf = (tempf/1024.0) * 5000/10;

char temp[5];				// Variable used to store tempf string.

itoa(tempf, temp, 10);		// Converts tempf integer to string.

if (message_received)

{

message_received = false;	// Message received, print it.

printf("Received message: %s\n",nrf24_read_message());

_delay_ms(500);

status = nrf24_send_message(temp);	// Send message as response.

if (status == true) printf("Message sent successfully\n");

}

}

}

ISR(INT0_vect)	// Interrupt on IRQ pin.

{

message_received = true;

}

void print_config(void)

{

uint8_t data;

printf("Startup successful\n\n nRF24L01+ configured as:\n");

printf("-------------------------------------------\n");

nrf24_read(CONFIG,&data,1);

printf("CONFIG		0x%x\n",data);

nrf24_read(EN_AA,&data,1);

printf("EN_AA			0x%x\n",data);

nrf24_read(EN_RXADDR,&data,1);

printf("EN_RXADDR		0x%x\n",data);

nrf24_read(SETUP_RETR,&data,1);

printf("SETUP_RETR		0x%x\n",data);

nrf24_read(RF_CH,&data,1);

printf("RF_CH			0x%x\n",data);

nrf24_read(RF_SETUP,&data,1);

printf("RF_SETUP		0x%x\n",data);

nrf24_read(STATUS,&data,1);

printf("STATUS		0x%x\n",data);

nrf24_read(FEATURE,&data,1);

printf("FEATURE		0x%x\n",data);

printf("-------------------------------------------\n\n");

}

void adc_init (void)	// Sets up and enables ADC.

{

ADMUX = (0<<REFS1)|		// Reference Selection Bits.

(1<<REFS0)|			// AVcc - external cap at AREF.

(0<<ADLAR)|			// ADC Left Adjust Result.

(0<<MUX2)|			// Analog Channel Selection Bits.

(0<<MUX1)|			// ADC0 (PC0).

(0<<MUX0);

ADCSRA = (1<<ADEN)|	// ADC Enable.

(0<<ADSC)|			// ADC Start Conversion.

(0<<ADATE)|			// ADC Auto Trigger Enable.

(0<<ADIF)|			// ADC Interrupt Flag.

(0<<ADIE)|			// ADC Interrupt Enable.

(1<<ADPS2)|			// ADC Pre-scaler Select Bits.

(0<<ADPS1)|

(1<<ADPS0);

}

- SCHEMATICS

The following schematic from the slides describe the connections that are made relatively well.

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

The following screen shot shows that the student successfully sent and received temperature information through the RF module.

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

The following is a picture of the board setup:

- VIDEO LINKS OF EACH DEMO

https://youtu.be/B2bz1ifVJcg

- GITHUB LINK OF THIS DA

https://github.com/dsenda/Smiles/tree/master/DA5

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Daniel Senda