CPE301 – SPRING 2019

# Design Assignment DA6

Student Name: Daniel Senda

Student #: 5002362633

Student Email: sendad1@unlv.nevada.edu

Primary Github address: https://github.com/dsenda/Smiles

Directory: DA6

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used:

Atmel Studio 7

ATmega328P Xplained Mini

MPU6050 (Gyroscope, Accelerometer, Temperature) Sensor

FTDI Basic chip

Breadboard

Jumper wires

Block diagram about the transmission connections:

ATmega328

MPU6050 Sensor

ATmega328p

FTDI chip

- INITIAL/MODIFIED CODE FROM TASK DA6

Not applicable, there is no modified code.

- DEVELOPED MODIFIED CODE OF TASK DA6

// c_code_MPU6050_gyro_accel.c

// Daniel Senda

// Makes use of the MPU6050 (Gyroscope, Accelerometer, Temperature) Sensor.

#define F_CPU 16000000UL	// Defines CPU clock frequency.

// Included libraries:

#include <avr/io.h>

#include <util/delay.h>

#include <inttypes.h>

#include <stdlib.h>

#include <stdio.h>

#include "inc\MPU6050_Res_DefineFile.h"

#include "inc\I2C_MasterFileH.h"

#include "inc\USART_RS232File_H.h"

// Variables:

char buffer[20], float_[10];

float Accel_x,Accel_y,Accel_z;

float Gyro_x,Gyro_y,Gyro_z;

// Functions:

void MPU6050_Init(void);

void values_in(void);

void MPU_Start_Loc(void);

void value_calcs(void);

void values2usart(void);

int main()

{

I2C_Init();			// Calls the I2C_Init function.

MPU6050_Init();		// Initializes MPU6050 sensor.

USART_Init(9600);		// Initializes USART.

while(1)

{

values_in();		// Reads values in from sensor.

value_calcs();	// Calculates sensor values.

values2usart();	// Sends calculated values to USART.

_delay_ms(1000);

}

}

void MPU6050_Init()			// Gyro initialization function.

{

_delay_ms(150);		// Power up time >100ms.

I2C_Start_Wait(0xD0);	// Starts at device that will be written to address.

I2C_Write(SMPLRT_DIV);	// Writes to sample rate register.

I2C_Write(0x07);		// Sets 1KHz sample rate.

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(PWR_MGMT_1);	// Writes to power management register.

I2C_Write(0x01);		// X axis gyroscope reference frequency.

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(CONFIG);		// Writes to configuration register.

I2C_Write(0x00);		// Fs = 8KHz.

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(GYRO_CONFIG);	// Writes to Gyroscope configuration register.

I2C_Write(0x18);		// Full scale range +/- 2000 degree/C

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(INT_ENABLE);	// Writes to interrupt enable register.

I2C_Write(0x01);

I2C_Stop();

}

void values_in()			// Reads values from sensor.

{

MPU_Start_Loc();

Accel_x = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Accel_y = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Accel_z = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Gyro_x = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Gyro_y = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Gyro_z = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Nack());

I2C_Stop();

}

void MPU_Start_Loc()

{

I2C_Start_Wait(0xD0);	// I2C starts with device write address.

I2C_Write(ACCEL_XOUT_H);	// Writes start location address from where to read.

I2C_Repeated_Start(0xD1);	// I2C starts with device read address.

}

void value_calcs()

{

// Calculates accelerometer values into G force units.

Accel_x = Accel_x/16384.0;

Accel_y = Accel_y/16384.0;

Accel_z = Accel_z/16384.0;

// Calculates gyroscope values into degrees/second units.

Gyro_x = Gyro_x/16.4;

Gyro_y = Gyro_y/16.4;

Gyro_z = Gyro_z/16.4;

}

void values2usart()

{

// Sends values to USART.

dtostrf( Accel_x, 3, 2, float_ );

sprintf(buffer," Accel_x = %s g\t",float_);

USART_SendString(buffer);

dtostrf( Accel_y, 3, 2, float_ );

sprintf(buffer," Accel_y = %s g\t",float_);

USART_SendString(buffer);

dtostrf( Accel_z, 3, 2, float_ );

sprintf(buffer," Accel_z = %s g\t",float_);

USART_SendString(buffer);

dtostrf( Gyro_x, 3, 2, float_ );

sprintf(buffer," Gyro_x = %s%c/s\t",float_,0xF8);

USART_SendString(buffer);

dtostrf( Gyro_y, 3, 2, float_ );

sprintf(buffer," Gyro_y = %s%c/s\t",float_,0xF8);

USART_SendString(buffer);

dtostrf( Gyro_z, 3, 2, float_ );

sprintf(buffer," Gyro_z = %s%c/s\r\n",float_,0xF8);

USART_SendString(buffer);

}

- SCHEMATICS

The following schematic describes the connections that are made relatively well.

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

The following screen shot shows that the student successfully set up the sensor.

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

The following is a picture of the board setup:

- VIDEO LINKS OF EACH DEMO

https://youtu.be/LO4jahOpe4A

- GITHUB LINK OF THIS DA

https://github.com/dsenda/Smiles/tree/master/DA6

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Daniel Senda